Computer Architecture and Organization (CSC3750)

Project 01

The assignment is a multipart assignment. The overall goal is to develop a virtual computer (VC) that can execute a simple set of machine-level program instructions.

# Part 1: Instruction Set Architecture Definition

# & Virtual Computer Assembler

In this part of the assignment, your task is to implement an assembler that can translate assembly-level instructions into virtual-computer machine instructions (VCI). The VC assembly-level instructions define the VC’s ISA (Instruction Set Architecture). The VC register set and ISA are defined below, along with the VC instruction formats. Use Visual Studio 2019 with C# to implement the assembler as a console-based application.

# Virtual Computer Register Set

- ACC – 32-bit Accumulator

- REG_A – 32-bit General Purpose Register

- REG_B– 32-bit General Purpose Register

- REG_C – 32-bit General Purpose Register

- REG_D – 32-bit General Purpose Register

- CMP – Stores the result of a comparison operation (0, 1, -1)

- PC – Program Counter

# Virtual Computer Instruction Set Architecture (ISA)

## Special Instructions

- HALT – Transfers control to the operating system

## Clear Instructions

- CLR ACC – Sets ACC to 0

- CLR REG_A – Sets REG_A to 0

- CLR REG_B – Sets REG_B to 0

- CLR REG_C – Sets REG_C to 0

- CLR REG_D – Sets REG_D to 0

## Add Instructions

- ADD REG_A – Add the 32-bit integer value stored in REG_A to ACC

- ADD REG_B – Add the 32-bit integer value stored in REG_B to ACC

- ADD REG_C – Add the 32-bit integer value stored in REG_C to ACC

- ADD REG_D – Add the 32-bit integer value stored in REG_D to ACC

## Move Instructions

- MOV ACC REG_A – Moves a 32-bit integer value from REG_A to ACC

- MOV ACC REG_B – Moves a 32-bit integer value from REG_B to ACC

- MOV ACC REG_C – Moves a 32-bit integer value from REG_C to ACC

- MOV ACC REG_D – Moves a 32-bit integer value from REG_D to ACC

- MOV REG_A ACC – Moves a 32-bit integer value from ACC to REG_A

- MOV REG_B ACC – Moves a 32-bit integer value from ACC to REG_B

- MOV REG_C ACC – Moves a 32-bit integer value from ACC to REG_C

- MOV REG_D ACC – Moves a 32-bit integer value from ACC to REG_D

- MOVI REG_A VALUE – Moves a literal 32-bit integer value into REG_A

- MOVI REG_B VALUE – Moves a literal 32-bit integer value into REG_B

- MOVI REG_C VALUE – Moves a literal 32-bit integer value into REG_C

- MOVI REG_D VALUE – Moves a literal 32-bit integer value into REG_D

## Store Instructions

- STORE REG_A ADDR – Stores a 32-bit integer value from REG_A to the memory location defined by ADDR

- STORE REG_B ADDR – Stores a 32-bit integer value from REG_A to the memory location defined by ADDR

- STORE REG_C ADDR – Stores a 32-bit integer value from REG_A to the memory location defined by ADDR

- STORE REG_D ADDR – Stores a 32-bit integer value from REG_A to the memory location defined by ADDR

## Load Instructions

- LOAD REG_A ADDR – Loads a 32-bit integer value from the memory location defined by ADDR to REG_A

- LOAD REG_B ADDR – Loads a 32-bit integer value from the memory location defined by ADDR to REG_B

- LOAD REG_C ADDR – Loads a 32-bit integer value from the memory location defined by ADDR to REG_C

- LOAD REG_D ADDR – Loads a 32-bit integer value from the memory location defined by ADDR to REG_D

## Compare Instructions

- CMP REG_A – Compares the 32-bit integer value stored in REG_A to ACC

- CMP REG_B – Compares the 32-bit integer value stored in REG_B to ACC

- CMP REG_C – Compares the 32-bit integer value stored in REG_C to ACC

- CMP REG_D – Compares the 32-bit integer value stored in REG_D to ACC

# Virtual Computer Instruction Format (Opcodes, Machine Code)

- HALT – 0x2000

- CLR ACC – 0x1000

- CLR REG_A – 0x1001

- CLR REG_B – 0x1002

- CLR REG_C – 0x1003

- CLR REG_D – 0x1004

- ADD REG_A – 0x0001

- ADD REG_B – 0x0002

- ADD REG_C – 0x0003

- ADD REG_D – 0x0004

- MOV ACC REG_A – 0x0101

- MOV ACC REG_B– 0x0102

- MOV ACC REG_C– 0x0103

- MOV ACC REG_D– 0x0104

- MOV REG_A ACC– 0x0201

- MOV REG_B ACC – 0x0202

- MOV REG_C ACC – 0x0203

- MOV REG_D ACC – 0x0204

- MOVI REG_A VALUE – 0x0601 IMMEDIATE_VALUE

- MOVI REG_B VALUE – 0x0602 IMMEDIATE_VALUE

- MOVI REG_C VALUE – 0x0603 IMMEDIATE_VALUE

- MOVI REG_D VALUE – 0x0604 IMMEDIATE_VALUE

- STORE REG_A ADDR – 0x0301 ADDR

- STORE REG_B ADDR – 0x0302 ADDR

- STORE REG_C ADDR – 0x0303 ADDR

- STORE REG_D ADDR – 0x0304 ADDR

- LOAD REG_A ADDR – 0x0401 ADDR

- LOAD REG_B ADDR – 0x0402 ADDR

- LOAD REG_C ADDR – 0x0403 ADDR

- LOAD REG_D ADDR – 0x0404 ADDR

- CMP REG_A – 0x0501

- CMP REG_B – 0x0502

- CMP REG_C – 0x0503

- CMP REG_D – 0x0504

# Virtual Computer Assembler

Implement an assembler to assemble the VC ISA instructions into VC machine code, also called opcode (operation-code). Store the generated machine code as ASCII characters in a text file. Store each generated machine opcode on a separate line.  The assembler should be written using the C# programming language.

Here’s an example VC assembly program:

CLR ACC

MOVI REG_A 0x0000 0001

ADD REG_A

MOVI REG_B 0x0000 0002

ADD REG_B

MOV REG_C ACC

HALT

Here’s an example VC assembly program:

CLR ACC

LOAD REG_A 0x0400 0000

ADD REG_A

LOAD REG_B 0x0400 0004

ADD REG_B

MOVE REG_C ACC

STORE REG_C 0x0800 0000

HALT

# Part 2: Virtual Computer Program Loader & Step-Through Debugger

In this part of the assignment, your task is to implement a VC Program Loader and a Step-Through Debugger. Use Visual Studio 2019 with C# to implement the Program Loader and Step-Through Debugger as a Windows-based Graphical User Interface (GUI) application. The GUI should include TextBoxes to display the Register values and a GridView to display the VC memory. Values in memory should be stored in an ASCII text file with each memory location/memory value pair on a line together. This ASCII text file represents the VC Virtual Memory component.

# Virtual Computer Virtual Memory ASCII File Example

0x0040 0000 0x0000 0001

0x0040 0002 0x0000 0002